| Exercise : 1 |  | Using RDODDA : # of Sources option |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Aim: |  | In this exercise RDODDA will be used to extract to the output file "Sky.dat" the Day number, time, average sounding frequencing , group range, amplitude, and number of sources for individual subcases, or over a group of subcases. Note Computer generated responses are given in Red, the required typed inputs are given in black, Comments are preceded by // and are in green. The <function> qualifier specifies keyboard function keys, e.g. |  |  |  |
|  |  | <Enter> <Esc> <F1> |  | Refers to the enter or return key Refers to the escape key Refers to the F1 key … etc….. |  |
| Example: |  | Remember before starting the program, delete any Sky.dat file that exists, or rename it for archiving the data set. Otherwise RDODDA will not allow you to overwrite this file and report an error if this file exists. In the DOS (VM) window, or from the command line enter the following; C:\ \DDA>rdodda <Enter> // Start program Enter path and file name. drift0.sky <Enter> // enter filename containing skymap data Specify Skymap=1, DDA.Vel=2, Q.C.=3 or GDDA.Vel=4 ?? 1 <Enter> # of sources = 1, edit skymap file = 0, ?? 1 <Enter> ENTER # OF SUBCASES GROUPING 1 <Enter> Stop - Program terminated. |  |  |  |
| Result : |  | The file Sky.dat will be generated with the following format. DAY,TIME,AVE.FRQ,AVE.HGT,AVE.AMP,AVE.SRC 10 20.10 3.71 90.00 55.50 25 10 20.10 3.71 95.00 48.00 23 10 20.10 3.71 100.00 49.50 21 10 20.10 3.71 105.00 49.50 31 10 20.10 3.71 110.00 63.00 36 |  |  |  |
|  |  | where | Day |  | Refers to Day number |
|  |  |  | FRQ.AVE |  | Refers to the mean of the sounding frequencies (in MHZ) contained in # of sub-case groupings. For one sub-case grouping this value will be the actual frequency sounded on in one Frequency/Range/Polarization measurment. |
|  |  |  | AVE.HGT |  | Refers to the mean of the group ranges (in km) contained in # of sub-case groupings. For one sub-case grouping this value will be the actual range sampled in one Frequency/Range/Polarization measurment. |
|  |  |  | AVE.AMP |  | Refers to the mean of the maximum amplitude (in dB) contained in # of sub-case groupings. For one sub-case grouping this value will be the average amplitude over the # of antennas used in one Frequency/Range/Polarization measurment. |
|  |  |  | AVE.SRC |  | Refers to the total number or sources contained in # of sub-case groupings. For one sub-case grouping this value will be the # of sources detected used in one Frequency/Range/Polarization measurment. |